FAST NUCES

National University of Computer and Emerging Sciences School of Computing, Karachi Campus

DIGITAL COMBINATION LOCK SYSTEM

Digital Logic Design Lab

----Team Members----

Khunais Baig 25K-0508

Yash Raj 25K-0878

Instructor: Ms. Seema Hashwani Section: BCS-2C

Contents

1.	Introduction	3

2.	Simulation Environment	3

3.	Components Used in the Circuit	4

4.	How the Circuit Works	6

4.1	User Input — Entering a Digit	6

4.2	Displaying and Latching	6

4.3	Latching the Signal	6

4.4	Comparing Against the Secret Code	7

4.5	Controlling the Lock State	7

4.6	Visual Output — LEDs	7

5. Signal Flow Summary	7

7. Conclusion	8

# Introduction

This project builds a Digital Combination Lock (DCL) using standard logic components, simulated in LogicWorks. Instead of writing software on a

microcontroller, the entire lock mechanism is built from hardware chips — counters, latches, a comparator, and logic gates — wired together to check a 4-digit PIN and

control the output.

Building a lock purely from hardware is a great way to understand the real foundations of

Digital Logic Design: how data is stored, compared, decoded, and acted upon without any code.

The system is designed around one complete digital block consisting of:

- A decade counter (74_192) driven by a push button, letting the user cycle through digits 0-9

- A 4-bit latch (74_75) that holds the entered digit steadily for comparison

- A BCD-7-Segment decoder (74_47) that drives a 7-segment display so the user can see the digit

- A magnitude comparator (74_85) that checks the entered digit against the stored secret code

- A D-Flip Flop and AND Gate network that controls the final clock start

# Simulation Environment

The circuit was designed and tested using LogicWorks. The project file (CombinationLock.cct) contains the complete schematic with all components. In the simulation, sub-circuit blocks like 192PLA and 75LATCH representing the 74-series chips are used to verify logic behavior.

# Components Used in the Circuit

| Component | Qty | Real-World IC | Role in the Circuit |
| --- | --- | --- | --- |
| 74_192 | 1 | 74LS192 | Decade up/down counter. Driven by the SPDT push-button to let the user scroll through digits 0–9. |
| 74_75 | 2 | 74LS75 | 4-bit latch. Holds the current counter output as a stable signal for the comparator and display driver. Two instances used for data stability. |
| 74_47 | 1 | 74LS47 | BCD-to-7-segment decoder. Converts the 4-bit binary counter value into segment drive signals for the 7-segment display. |
| 74_85 | 1 | 74LS85 | 4-bit magnitude comparator. Compares the user-entered digit (Port A) against the hardwired secret code (Port B). Outputs A=B when they match. |
| D Flip Flop | 1 | Generic DFF | Stores the lock state (locked/unlocked). Clocked to capture the comparator output and hold it until cleared. |
| and_4 | 1 | 4-input AND | 4-input AND gate. Used to combine multiple match or control signals into a single decision output. |
| and_2 | 1 | 2-input AND | 2-input AND gate. Used in the enable/control logic chain. |
| AND-2 (1-Inv) | 1 | AND with inverted input | AND gate with one inverted input (effectively NAND on one side). Used for reset/clear control logic. |
| NOT | 1 | 74LS04 (1 gate) | Inverter. Used to negate a control signal in the logic chain. |
| 7-Seg Disp - Black | 1 | Common-anode 7-segment | LED display. Shows the current digit the user has selected (0–9). |
| LED - Green | 1 | Green LED | Lights up when the entered combination is correct (Access Granted). |
| LED - Red | 1 | Red LED | Lights up when the combination is wrong or the system is in locked state (Access Denied). |
| SPDT Pushbutton | 1 | Push-button switch | The user-facing input. Each press steps the counter up or down to change the digit. |
| Binary Switch | 1 | DIP switch bank | Sets the secret combination code. The 4-bit output is wired directly to Port B of the magnitude comparator. |
| PULLUPB | 1 | Pull-up resistor | Keeps the push-button input line HIGH when the button is not pressed, preventing floating/undefined input. |
| TSBUF4 | 1 | Tri-state buffer (4-bit) | Controls when the counter output is passed to the rest of the circuit, preventing signal conflicts. |
| OCBUF | 1 | Open-collector buffer | Provides an open-collector output for driving the final output stage cleanly. |
| Power / VCC | Multiple | VCC rail | Provides the logic HIGH supply (5V) to all chips and pull-up resistors. |
| GND | Multiple | Ground rail | Common ground reference for the entire circuit. |
| Port In / Out / Bidir | Multiple | Interface ports | LogicWorks simulation ports used to define inputs (push-button, secret code) and outputs (display, LEDs) at the boundary of the design. |

# How the Circuit Works

## User Input — Entering a Digit

The user interacts with an SPDT push-button. Each press clocks the 74_192 decade counter, stepping the value up (or down). The counter cycles through 0 to 9 in BCD format. A pull-up resister (PULLUPB) keeps the clock line stable.

A tri-state buffer (TSBUF4) gates the counter output to the rest of the circuit, ensuring clean signal propagation only when needed.

## Displaying and Latching

The 4-bit output is decoded by the 74_47 for the display and held steady by two 74_75 latches to prevent input glitches during comparison.

## Latching the Signal

Two 74_75 4-bit latches hold the counter’s output as a stable value. Without latching, brief glitches or bounces from the push-button could momentarily corrupt the signal going to the comparator. The latches act as a buffer, ensuring the comparator always sees a clean, steady input.

## Comparing Against the Secret Code

The stable, latched digit is presented to the A-port of the 74_85 magnitude comparator. The B-port is connected to the Binary Switch, which holds the secret combination code as a fixed 4-bit value.

The 74_85 compares both values bit by bit. If every bit matches, it asserts the A=B output HIGH. If even one bit differs, A=B stays LOW. The cascade inputs (A>Bin, A<Bin, A=Bin) are set to standard values (A=Bin = HIGH) to configure it as a standalone comparator.

## Controlling the Lock State

The A=B output from the comparator flows into the AND gate network (and_4, and_2, and AND-2 with 1-Inv input), which combines the match signal with any enable or reset conditions. The output of this logic drives the clock input of the D Flip Flop.

The D Flip Flop stores the final lock decision:

- When a correct match is clocked in, the flip flop output goes HIGH (unlocked state).

- When reset or a mismatch occurs, the flip flop is cleared back to LOW (locked state).

A NOT gate inverts one of the control signals to generate the complementary reset/enable path.

## Visual Output — LEDs

The Q output of the D Flip Flop drives both LEDs:

- Green LED lights up when Q is HIGH — the code matches, access is granted.

- Red LED lights up when Q is LOW — the code is wrong or the system is locked.

An open-collector buffer (OCBUF) sits in the output path to provide clean, current-limited drive to the LEDs without loading the flip flop directly.

# 5. Signal Flow Summary

| Step | Component | Action |
| --- | --- | --- |
| 1 | SPDT Pushbutton + PULLUPB | User presses button → clock pulse generated |
| 2 | 74_192 (via 192PLA) | Counter steps up/down through 0–9 |
| 3 | TSBUF4 | Counter output gated to the circuit |
| 4 | 74_75 (via 75LATCH) ×2 | BCD value latched into stable storage |
| 5 | 74_47 (via 46LOGIC) | BCD decoded → 7-segment display shows digit |
| 6 | 74_85 (via 85PLA) | Latched digit compared to Binary Switch secret code |
| 7 | and_4 / and_2 / AND-2 (1-Inv) / NOT | Match signal processed through control logic |
| 8 | D Flip Flop | Lock state stored (HIGH = unlocked, LOW = locked) |
| 9 | OCBUF | Output buffered for LED drive |
| 10 | Green / Red LED | Access Granted (green) or Denied (red) displayed |

# 7. Conclusion

The Digital Combination Lock project successfully demonstrates all the key pillars of Digital Logic Design using a LogicWorks simulation. A user-driven counter, a visual display, a hardware comparator, sequential state storage in a flip flop, and dual LED output together form a working authentication system — built entirely from logic gates and standard 74-series components.

The circuit accurately reflects what would be constructed on a real breadboard or PCB, with the LogicWorks sub-circuit models (192PLA, 85PLA, 46LOGIC, 75LATCH) faithfully representing the real 74LS192, 74LS85, 74LS47, and 74LS75 chips. This project confirms that even complex security behavior can emerge from simple, well-understood digital building blocks.